ATMEGA BOOTLOADER TUTORIAL

Before starting this tutorial, please read my bootloading tutorial (coming soon!) and my UART tutorial! You must also already have a UART connection set up on your robot for this to work: $50 Robot UART tutorial.

As you should know, when I finish my other bootloading tutorial, is that bootloaders are software that can replace your hardware programmer. Instead of hooking up a programmer, you can program using a serial connection. You will need a programmer to upload the bootloader, but you won't ever need the programmer again, except for maybe programming fuses or lockbits.

Now that you understand what a bootloader is and the benefits for one, now I will demonstrate how to install a bootloader onto your $50 Robot or any other robot with an ATmega microcontroller.

We will be using a bootloader that has an auto-baud feature, where your microcontroller will attempt to reconfigure its internal baud settings to ensure proper bootloader configuration. This does not mean your other hardware will auto-configure, soooo . . .

Important: Make sure that all your external hardware is set to the same exact baud rate or this will not work!

Just for reference, the bootloader I selected is the open source fast tiny & mega UART bootloader. This bootloader is a bit buggy, comes with zero documentation, and not much in comments in the source code . . . but it's the best I've found that can handle a wide range of ATmega types. I've made some small config changes to it for use on the $50 Robot, and those files will be downloadable later in this tutorial.

Configure BAUD (if you haven't already done so)

Click: 
Start->Settings->Control Panel->System

A new window will come up called 'System Properties'. Open the Hardware tab and click device manager. You should see this:

Go to Ports, select the one you are using, and right click it.

Select Properties. A new window should come up, and select the Port Settings tab:

Now configure the settings as you like, as described in the UART tutorial.

Upload Bootloader

You have two options here. You can either use my precompiled bootloader:

ATmega8 Bootloader hex file 
ATmega168 Bootloader hex file 
Axon USB Bootloader hex file 
beta v2.1 Bootloader files

and then upload it using AVR Studio.

Or you can custom modify it for your specific setup/ATmega. In the following steps I'll explain both.

update: new bootloader software is available

Programming your own Bootloader

Note: If you do not plan to modify the bootloader code, you may skip this step.

Open up AVR Studio, and in the Project Wizard start a new project called bootloader:

Make sure you select 'Atmel AVR Assembler' since we will be programming in Assembly. Don't worry, its mostly done for you already. You do not need to program in Assembly to write a bootloader, but the particular bootloader we are using is written in that language, and so we must compile for it.

Click Finish, and the new project should load up.

Install Files

Now, download this zip file, and unzip it into your bootloader directory:

Bootloader Source Files (v1.9) 
Bootloader Source Files (v2.1)

note: this tutorial teaches only v1.9, but v2.1 is better

Now you must also put your own robot program .hex into the bootloader file as well. For example, suppose you just modified your own custom photovore code and you compiled it. Take that compiled .hex and place it into your bootloader folder. Don't forget to do this every time you change your custom code!

Optional: Compile Code 
Note: If you do not plan to modify the bootloader code, you may skip this step.

Look for a file that matches the microcontroller you are using. For example, if you are using the ATmega168, look for the file M168.asm. Open that file up, and copy/paste the contents into your bootloader.asm that is already open in AVR Studio.

Now looking at the datasheet of your microcontroller (pin out section), verify that the Tx and Rx pins are correct in bootloader.asm. This is an important step, and in rare cases can break something if you skip this step!!! Make any changes as needed.

For example, this is what it should look like for both the ATmega8 and ATmega168:

.equ    STX_PORT        = PORTD

.equ    STX_DDR         = DDRD

.equ    STX             = PD1

.equ    SRX_PIN         = PIND

.equ    SRX_PORT        = PORTD

.equ    SRX             = PD0

Now compile it by pressing build:

Upload Code to ATmega

Now that you have your new custom bootloader .hex file, you need to simply upload that to your microcontroller. Use your hardware programmer like you have always done:

And finally, you need to program a fuse to tell it to use the bootloader.

IMPORTANT: If you change the wrong fuse you can possibly destroy your ATmega! Don't change any other fuses unless you know what you are doing!

You want to set BOOTRST to 0 by checking this box, and then pushing Program:

Your Bootloader is Uploaded and Ready!

Now disconnect your programmer cable. You won't be needing that again!

You will need to power cycle your microcontroller (turn it off then on again) after uploading your bootloader for the settings to take effect.

Upload YOUR Program Through UART

update 2010: A GUI version of the bootloader can be found on the Axon II setup tutorial.

Now open up a command prompt by going to

start->Run...

and typing in 'cmd' and pushing ok:

A new command prompt should open up. Using the command 'cd', go into the directory of your bootloader files. See below image for an example.

With your robot turned off and UART ready to go, type in this command: 
fboot17.exe -b38400 -c1 -pfile.hex -vfile.hex

38400 is your desired baud (9600, 38400, 115200, etc)

c1 is your com port (c1, c2, c3, etc)

'file' is the name of your program you want uploaded. The filename MUST be 8 characters or less or it will not work (a bug in the software), and the file must be located in the same folder as fboot.exe. For example, if photovore.hex was your file, do this: 
-pphotovore.hex -vphotovore.hex 
(yes, you need to say it twice, with p for the first time and v for the second time)

Press enter, and now you will see a / symbol spinning. Turn on your robot, and it should now upload.

This is what you should see upon a successful bootload:

For some unexplained reason, I occasionally get an error that says: 
Bootloader VFFFFFFFF.FF 
Error, wrong device informations

If you get this error, just repeat this step again and it should work.

note: after typing in a command once into the command prompt, you do not need to type it again. Just push the up arrow key to cycle through previously typed commands.

The Bootloader Didn't Work?!

What if I did the tutorial but its still not working/connecting?

Chances are you missed a step. Go back to the beginning and make sure you did everything correctly.

Try power cycling your microcontroller.

Make sure the hardware programmer is unplugged.

Make sure baud is configured properly on ALL of your hardware and ALL of your involved software.

Make sure no other device is trying to use the same com port at the same time, such as AVR Studio, HyperTerminal, etc.

Some mistakes that you can make will cause your command prompt window to freeze up. Just open up a new window and try again.

Some users have noticed that too many unused globabl variables in your source code will cause problems. See this forum post for more info.

And a side note . . . this bootloader can only connect with com ports 1 to 4. The developer of the bootloader for some odd reason thought there isn't anything wrong with this decision . . . If you need a different port, go to the com port settings and change the port you are using.

Also to note, some of your UART hardware might not be fast enough as the software doesn't wait for hardware to keep up. The Easy Radio module will not work, for example. A direct serial/USB connection will work without a problem.

GETTING STARTED
The Bootloader

Part #1: Installing Software 
Part #2: Setting up Hardware 
Part #3: Using the Bootloader

This tutorial will show you how to upload programs to your Axon using a bootloader.

note: If you are using Linux or Mac, please see the FAQ

note: Your system must be 32-bit to use the bootloader (if you don't, use a virtual machine, or a hardware programmer).

note: If you already own a hardware programmer, using the bootloader is optional. Simply plug in the ISP header and program the Axon II directly. The bootloader is just there to save you the $36 of buying a hardware programmer. The hardware programmer will overwrite the bootloader - but you can always reupload the bootloader again later.

note: The following demonstrates how to use the GUI version of fboot. If you prefer to use the classic command line version, see the old fboot setup tutorial. It will also show you where to download source for it, and a tutorial on how to modify it.

note: Thanks to catalinene33 for making the GUI!

STEP 1: Download Bootloader GUI Download and install the .exe into the same folder that you have your compiled robot code.

32 bit Windows:

FBOOT GUI (300kb)

Unzip, and run the program.

64 bit Windows:

McUber bootloader (beta)

Linux and Mac:

It is highly recommended to use a Windows 32 bit VR machine, but you may also try this bootloader.

STEP 2: Select Options

Select baud rate. 
115200 is your desired baud. The bootloader has an 'autobaud' feature, so it's likely to work under most baud settings. Try slower rates if it has problems on your system.

Select com port. 
c1 is your Axon II USB COM port (c1, c2, c3, etc) as shown in your Device Manager (see step 3). The bootloader only allows c1 to c4.

Select .hex file. 
'file' is the name of your program you want uploaded.

STEP 3: Run Bootloader

Make sure your Axon USB is plugged in, and power turned off.

Press Flash, and a new command prompt window will open with a \ symbol spinning.

Turn your Axon on, and the command prompt window should then look something like this:

To use the GUI again, just exit the command prompt window.

note: Remember, the bootloader must always be activated BEFORE you turn on the Axon or it will not work.

note: if it takes more than about ~8 seconds to load your code, please see the FAQ on how to speed up your bootloader.

It didn't work? Read the bootloader debugging checklist.

STEP 4: Be Happy!

Congrats, you just programmed your Axon II successfully!

The next step would be to write your own code. Refer back to STEP 4: Setup Project with Project Designer for programming help.

If you want free webspace to post your project, check out the SoR Member Pages. If you make anything neat with the Axon, or want others to see your work, send me a heads up and I'll post your project on the Axon Examples.

# fast tiny & mega UART bootloader

After every power on reset, the bootloader wait for about 0.3 seconds for a keyword to enter the bootloader, otherwise the application was started. So it is also possible to use the reset pin as IO on small Attiny or ATmega.

There is no need for a crystal, since autobauding was done prior checking the keyword.
On frequencies below 8MHz, programming at 115200Baud may fail and you should try a lower baudrate. Baudrates down to 1200baud can be selected. There is a small download program included, which can easy be executed on the end of the make file.
"fboot.exe -?" tells the usage.
It must be started prior powering or resetting the AVR.


There are two different programming algorithm implemented:
1.
The ATtiny and ATmega without bootstart fuses need to place the RJMP instruction at first. This was replaced by the RJMP to the bootloader and recalculated to be placed one word below the bootloader.
Especially care was taken against accidentally lockout. Thus erasing was done backward with erasing the page with the bootloader jump at last. So if power fails unfortunately at this moment, all pages are erased to 0xFFFF, which was interpreted like a NOP and thus the bootloader was entered anyway.

2.
On Atmega equal and above 8kB the bootstart fuse must be set to enter the bootloader. 
For faster download a buffersize bigger than the pagesize was used.


Following devices are tested:

AVR---------user size---programming time
---------------------------------------------
ATtiny13----606 Byte----0.77 seconds
ATtiny2313--1630 Byte---2.86 seconds
ATtiny45----3646 Byte---3.52 seconds
ATmega8-----7680 Byte---1.81 seconds
ATmega16----15872 Byte--3.35 seconds
ATmega32----32256 Byte--5.33 seconds
ATmega644---64512 Byte--8.19 seconds
ATmega2561--261120 Byte--32.75 seconds

Now updated to Version V2.1
An API-call was implemented to program a Flash page from the application.
The example "apicall.c" was written for AVR-GCC (WINAVR).
Error correction for ATmega32 (wrong start address)